LED Integrated Testing System test report

| test item: | LED spectrum analyse |  |  |
| --- | --- | --- | --- |
| equipment: | Led test system |  |  |
| 设备厂商获得证书： | 国家半导体工程研发及产业联盟证书，中国光学光电子行业协会光电器件分会证书，高新技术企业证书 |  |  |
|  | Mode:MB 030 NRA 20061030 6000K | Manufacturer: |  |
| Test Identifier | Temperature:25℃ | Humidity:60% |  |
|  | Conner: | Date:2017.01 |  |
|  | Organ: |  |  |

curve of spectrum power distributing

| Spectrum |  |  | Electrical |  |  |
| --- | --- | --- | --- | --- | --- |
| λ(Peak): | 451.5 | nm | I(test): | 260.000 mA |  |
| λ(Main): | 489.7 | nm | Vf: | 220.100 V |  |
| λ(Centroid): | 0.0 nm |  | φv: | 2388.00 lm |  |
| λ(Center): | 0.0 nm |  | Efficiency: | 78.812 lm/w |  |
| BandWidth: | 24.7 | nm | P: | 30.3 W |  |
| ColorTemp: | 6290 | K | PFC: | 0.528 |  |
| CIE(x,y): | 0.3110, 0.3299 |  | Ra1:82.2 | Ra2:89.0 | Ra3:93.5 |
| CIE(u,v): | 0.1963, 0.3124 |  | Ra4:84.4 | Ra5:83.6 | Ra6:84.7 |
| Ra: | 84.3 |  | Ra7:87.2 | Ra8:69.7 | Ra9:12.4 |
| Purity: | 0.078 |  | Ra10:73.7 | Ra11:86.3 | Ra12:57.2 |
| Light Power: | 6381.8966 mw |  | Ra13:83.8 | Ra14:96.7 | Ra15:76.9 |

Remark:

page 1 of 1	2016-11-10	16:32:14